Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

Aim

To demonstrate the OSI and TCP/IP network communication models using a network simulation tool and observe how data moves across each layer.

Objective

- Understand the layers of the OSI and TCP/IP models.

- Build a simple network topology in a simulation tool.

- Observe packet flow and encapsulation across layers.

- Compare the OSI model with the TCP/IP model.

Selected Tool

Cisco Packet Tracer (free for learners; alternatives: GNS3, Wireshark with a virtual lab)

Overview

A network simulation tool lets us create virtual devices (PCs, switches, routers, servers) and watch how data travels between them without physical hardware. Packet Tracer's Simulation Mode shows each PDU (Protocol Data Unit) passing through the layers of the OSI model.

OSI Model Layers

| Layer | Name | Function | Example Protocol / Device |
| --- | --- | --- | --- |
| 7 | Application | User interface to network services | HTTP, FTP, DNS, SMTP |
| 6 | Presentation | Data formatting, encryption, compression | SSL/TLS, JPEG |
| 5 | Session | Establish, manage and terminate sessions | NetBIOS, RPC |
| 4 | Transport | Reliable delivery, segmentation, ports | TCP, UDP |
| 3 | Network | Logical addressing and routing | IP, Router |
| 2 | Data Link | Physical addressing, framing | Ethernet, MAC, Switch |
| 1 | Physical | Transmission of raw bits | Cables, Hub, NIC |

TCP/IP Model Layers

| TCP/IP Layer | Equivalent OSI Layers | Protocols |
| --- | --- | --- |
| Application | Application, Presentation, Session | HTTP, DNS, FTP, SMTP |
| Transport | Transport | TCP, UDP |
| Internet | Network | IP, ICMP, ARP |
| Network Access | Data Link, Physical | Ethernet, Wi-Fi |

Experiment Setup

Network Topology

- 2 PCs (PC0, PC1)

- 1 Switch (Switch0)

- 1 Router (Router0) and a second LAN with 1 PC (PC2)

- 1 Web/DNS Server

IP Addressing Plan

| Device | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| Router (Fa0/0) | 192.168.1.1 | 255.255.255.0 | - |
| Router (Fa0/1) | 192.168.2.1 | 255.255.255.0 | - |
| PC2 / Server | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |

Procedure

Open Packet Tracer and place the devices on the workspace

↓

Connect devices using copper straight-through cables

↓

Assign IP addresses, subnet masks and gateways

↓

Verify connectivity using the ping command

↓

Switch to Simulation Mode and filter ICMP / HTTP events

↓

Send a PDU from PC0 to the server and inspect each layer

↓

Observe encapsulation and de-encapsulation at every device

Observations

Data Encapsulation

| Layer | Data Unit | Added Information |
| --- | --- | --- |
| Application | Data | Application data (e.g., HTTP request) |
| Transport | Segment | Source and destination port numbers |
| Network | Packet | Source and destination IP addresses |
| Data Link | Frame | Source and destination MAC addresses |
| Physical | Bits | Electrical / optical signals |

- The switch operates at Layer 2 and forwards frames using MAC addresses.

- The router operates at Layer 3 and forwards packets using IP addresses.

- ARP resolves an IP address to a MAC address before the first frame is sent.

- The first ping may fail due to ARP resolution, later pings succeed.

OSI vs TCP/IP Comparison

| Criteria | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of Layers | 7 | 4 |
| Developed By | ISO | US Department of Defense |
| Approach | Theoretical reference model | Practical, protocol-driven model |
| Usage | Teaching and troubleshooting | Used on the real Internet |
| Reliability | Transport layer only | Transport layer (TCP) |

Results

- The network was successfully simulated and end-to-end connectivity was verified using ping.

- Simulation Mode clearly showed encapsulation and de-encapsulation across layers.

- Switch and router behaviour matched their OSI layer roles (Layer 2 and Layer 3).

- TCP/IP was observed to map directly onto the OSI layers in a simplified form.

Conclusion

The experiment demonstrates how data travels through the layers of the OSI and TCP/IP models. A simulation tool such as Packet Tracer makes these abstract concepts visible and helps in understanding addressing, routing and troubleshooting in real networks.